| UNITED NATIONS |  | EP |
| --- | --- | --- |
|  |  | UNEP(DTIE)/Hg/OEWG.2/INF/14 |
|  | United Nations Environment Programme | Distr.: General 1 October 2008 English only |

Ad Hoc Open-ended Working Group on Mercury

Second meeting

Nairobi, Kenya

6–10 October 2008

Item 4 of the provisional agenda*

Report on activities under the UNEP mercury programme

Summary report on UNEP mercury inventory activities

Note by the secretariat

- 1. 	By paragraph 25 of decision 24/3 IV on chemicals management, the Governing Council of the United Nations Environment Programme (UNEP) requested the Executive Director to continue to facilitate work between the UNEP mercury programme and Governments, other international organizations, non-governmental organizations, the private sector and the partnerships established under the mercury programme, as appropriate, to improve global understanding of international mercury emissions sources, fate and transport and to promote the development of inventories of mercury uses and emissions.

- 2.	Annexed to the present note is a summary report on national results in developing mercury inventories, prepared by the Chemicals Branch of the UNEP Division of Technology, Industry and Economics. The document has been reproduced without formal editing.

Annex

Summary Report on UNEP Mercury Inventory Activities

Date:  1 October 2008

# Table of Contents

Overview	4

1.0  Summary of Current Results	5

1.1 – Country profiles: Population and GDP	5

1.2 – General correlation between population, GDP and Hg Inputs / Outputs	6

1.3 - Categories selected per country 	7

1.4 – Input Data

1.4.1 Summary of max and min Hg input per country

1.4.2 Max & min  Input data per category and per country	9

1.5  Output data 	13

1.5.1 General output data (min and max values)	13

1.5.2 Output data per country and  category 	14

1.5.3 Mass balance between input and output	20

2.0  Observations and lessons learned in applying the toolkit methodology	23

2.1  Understanding, Structure and Language	23

2.2  Toolkit Categories, Emission factors 	23

2.3  Data gathering	23

2.4  Double Recording	24

Table 1: Country profiles: Population and GDP	5

Table 2: Categories selected per country	7

Table 3: Summary of max and min Hg input per country	9

Table 4: Max & min  Input data per category and per country	10

Table 5: General output data (min and max values) 	13

Table 6: Output data per country and  category Philippines	14

Table 7: Output data per country and  category Yemen	15

Table 8: Output data per country and  category Cambodia	16

Table 9: Output data per country and  category Syria	17

Table 10: Output data per country and  category Burkina Faso	18

Table 11: Output data per country and  category Madagascar 	19

Table 12: Mass balance between input and output	20

Table 13: Category 55 : Consumer products containing mercury	24

Table 14: sub cat of cat 58 & 59	25

### Overview

UNEP Governing Council Decisions 23/9 and 24/3 call for work to be facilitated on the promotion and development of inventories of mercury uses and releases.

A key training and guidance document that supports countries efforts to take action on mercury is the ‘Toolkit for identification and quantification of mercury releases’.   The Pilot Toolkit was released in November 2005.

Activities to date have provided a start in identifying and quantifying mercury use and release at the national level while pilot testing the methodologies outlined within the toolkit.  In light of the experiences in using the toolkit, UNEP intends to revise the toolkit before the end of 2008.  Input into the toolkit update is welcomed at all times.  Input should be sent to:  mercury@chemicals.unep.ch  .

The attached report on UNEP mercury inventory activities is intended to provide a summary of the current results at the country level and outline observations and lessons learned in applying the toolkit methodology to date.

A number of countries are working with UNEP to develop national mercury inventories while applying the Toolkit methodology, including Cambodia, Pakistan, Philippines, Syria and Yemen that participated in a regional project in the Asian region, as well as Burkina Faso and Madagascar.

In addition, Chile, Ecuador and Panama have been working with the United Nations Institute for Training and Research to develop an inventory of releases with special consideration of products situation.

UNEP’s national inventory and follow-up action plan project results are available on the Fate and Transport partnership area web page on the UNEP Global Mercury Partnership web-site:

www.chem.unep.ch/mercury/partnerships/new_partnership.htm

Additional projects will be initiated as funds become available.

## 1.0  Summary of Current Results

The following results have been consolidated to provide the mercury open ended working group (OEWG) with an overview of the national mercury inventory results of national level projects undertaken with UNEP.    It has been compiled to provide a high level analysis of results.  It is recognized that national results will vary depending upon the country situation.

For specific detail at the national level, national project results are available on the Fate and Transport partnership area web page on the UNEP Global Mercury Partnership web-site:

www.chem.unep.ch/mercury/partnerships/new_partnership.htm

### 1.1 – Country profiles: Population and GDP

The population and GDP are indicators of both consumption level and economic level.

Table 1: Country profiles: Population and GDP

| Nom pays | population | GDP (billion $) |
| --- | --- | --- |
| Pakistan | 173 000 000 | 410 |
| Philippines | 96 000 000 | 299 |
| Madagascar | 20 000 000 | 18 |
| Syria | 19 747 000 | 87 |
| Yemen | 23 000 000 | 52 |
| Cambodge | 14 000 000 | 25,9 |
| Burkina Faso | 15 000 000 | 17,2 |

### 1.2 – General correlation between population, GDP and Hg Inputs / Outputs

The charts below show the general correlation between population, GDP and inputs / Output of mercury.

Graph 1: Inputs per country compared to Population and GDP

Graph 2: Output ¨per country compared to Population and GDP

### 1.3 - Categories selected per country

Codes : ( S: Syria; P: Philippines; Y: Yemen; C: Cambodge; B: Burkina, M: Madagascar)

The first step outlined within the toolkit is the identification of main source categories at the country level, facilitating preliminary evaluation of activities.  The following table highlights the categories identified by countries in applying the toolkit methodology.

The categories provide useful insight.  However, they do not illustrate information in terms of mercury input and output.

Table 2: Categories selected per country

| no | Sub category name | B | C | M | P | S | Y | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 511 | Coal combustion in large power plants |  |  | X | x |  |  | 2 |
| 512 | Other coal use | x |  |  |  |  |  | 1 |
| 513 | Mineral oils - extraction, refining and use | x | x | X | x | x | x | 6 |
| 514 | Natural gas - extraction, refining and use |  | x |  | x |  | x | 5 |
| 516 | Biomass fired power and heat production | x | x | X |  | x | x | 1 |
| 517 | Geothermal power production |  |  |  | x |  |  | 1 |
| 522 | Gold and silver extraction with mercury amalgamation processes | x | x | X | x |  |  | 4 |
| 524 | Copper extraction and initial processing |  |  |  | x |  |  | 1 |
| 525 | Lead extraction and initial processing |  |  |  | x |  |  | 1 |
| 529 | Primary ferrous metal production |  |  | X |  |  | x | 2 |
| 531 | Cement production (a |  |  | X | x | x | x | 4 |
| 532 | Pulp and paper production |  |  |  | x |  |  | 1 |
| 533 | Production of lime and light weight aggregates |  | x | x |  |  |  | 2 |
| 533 | Production of lime and light weight aggregates |  |  |  |  |  | x | 1 |
| 541 | Chlor-alkali production with mercury-technology |  |  |  |  | x |  | 1 |
| 551 | Thermometers with mercury | x | x | x | x | x | x | 6 |
| 552 | Electrical switches and relays with mercury |  |  | x | x |  | x | 3 |
| 553 | Light sources with mercury | x |  | x | x | x | x | 5 |
| 554 | Batteries with mercury | x | x | x |  | x | x | 5 |
| 555 | Biocides and pesticides with mercury |  |  | x |  |  |  | 1 |
| 556 | Paints with mercury |  |  | x |  |  | x | 2 |
| 557 | Production of pharmaceuticals for human and veterinary uses |  |  |  |  | x |  | 1 |
| 558 | Cosmetics and related products with mercury | x |  | x |  |  | x | 3 |
| 561 | Dental mercury-amalgam fillings (b | x | x | x | x | x | x | 6 |
| 562 | Manometers and gauges with mercury |  |  |  |  | x | x | 2 |
| 563 | Laboratory chemicals and equipment with mercury |  |  |  | x | x |  | 2 |
| 565 | Miscellaneous product uses, mercury metal uses, and other sources |  |  |  | x |  |  | 1 |
| 581 | Incineration of municipal/general waste |  | x |  |  |  |  | 1 |
| 583 | Incineration of medical waste | x | x | x |  | x | x | 5 |
| 585 | Informal waste incineration | x |  | x |  | x |  | 3 |
| 591 | Controlled landfills/deposits (a | x | x |  | x | x |  | 4 |
| 593 | Informal local disposal of industrial production waste |  |  | x | x |  |  | 2 |
| 594 | Informal dumping of general waste (b | x |  |  |  |  |  | 1 |
| 594 | Informal dumping of general waste (b |  |  |  |  |  | x | 1 |
| 595 | Waste water system/treatment | x |  | x | x |  |  | 3 |
| 5101 | Crematoria |  | x |  | x |  |  | 2 |
| 5102 | Cemetaries |  |  | x | x |  |  | 2 |

### 1.4  Input Data

1.4.1  Summary of max and min Hg input per country

The basic aim of the Toolkit is to enable an estimation of the average annual release to pathway (air, water, land, products, general waste, secto-specific waste treatment) for each release process identified.  The classification of maximum and minimum input per country comes from the use of input factors assigned to each category by default in the tool kit and which have been widely used by all countries at the stage of preliminary inventory.  Table illustrates that the input factor vary with geography.

Table 3: Summary of max and min Hg input per country

| max input factor |  |  | min input factor |  |  |
| --- | --- | --- | --- | --- | --- |
| Country | Input kg / year |  | Country | Input kg / year | Difference max /min |
| Philippines | 230 500 |  | Philippines | 131 932 | 98 568 |
| Madagascar | 82 949 |  | Madagascar | 27 951 | 54 998 |
| Syria | 24 796 |  | Syria | 10 457 | 14 339 |
| Cambodia | 14 854 |  | Yemen | 4 053 | 10 801 |
| Yemen | 5 770 |  | Cambodia | 2 072 | 3 698 |
| Burkina Faso | 4 565 |  | Burkina Faso | 1 238 | 3 327 |
| Total | 363 434 |  | Total | 177 703 | 185 731 |

The default input factors within the toolkit are preliminary and will be reviewed in future updates to the toolkit.

1.4.2  Max & min  Input data per category and per country

In developing national inventories, countries were encouraged to choose the input factor that seemed to best represent the sub-category under investigation.  It is understood that it may be appropriate to confirm this data for local/national conditions before major decisions are taken on implemnatioan of mitigation activities (particularly where there is large uncertainty in the maximum and minimum data).

Table 4: Max & min  Input data per category, per country

| No sub cat | Sub category name | Country | Max (kg) | Min (kg) |
| --- | --- | --- | --- | --- |
| 511 | Coal combustion in large power plants | Madagascar | 17 | 2 |
| 511 | Coal combustion in large power plants | Philippines | 5 386 | 539 |
| 512 | Other coal use | Burkina Faso | 132 | 13 |
| 513 | Mineral oils - extraction, refining and use | Philippines | 37 | 0,37 |
| 513 | Mineral oils - extraction, refining and use | Burkina Faso | 62 | 1 |
| 513 | Mineral oils - extraction, refining and use | Cambodia | 119 | 3 |
| 513 | Mineral oils - extraction, refining and use | Madagascar | 46 | 45 |
| 513 | Mineral oils - extraction, refining and use | Syria | 5 542 | 138 |
| 513 | Mineral oils - extraction, refining and use | Yemen | 5 | 1 564 |
| 514 | Natural gas - extraction, refining and use | Cambodia | 0,01 | 0,001 |
| 514 | Natural gas - extraction, refining and use | Philippines | 577 | 6 |
| 514 | Natural gas - extraction, refining and use | Syria | 3 320 | 33 |
| 516 | Biomass fired power and heat production | Syria | 0,0004 | 0,000 |
| 516 | Biomass fired power and heat production | Cambodia | 0,16 | 0,04 |
| 516 | Biomass fired power and heat production | Burkina Faso | 1 | 0,27 |
| 516 | Biomass fired power and heat production | Yemen | 3 | 3 |
| 516 | Biomass fired power and heat production | Madagascar | 182 | 182 |
| 517 | Geothermal power production | Philippines | 41 860 | 31 395 |
| 522 | Gold and silver extraction with mercury amalgamation processes | Madagascar | 180 | 60 |
| 522 | Gold and silver extraction with mercury amalgamation processes | Burkina Faso | 400 | 400 |
| 522 | Gold and silver extraction with mercury amalgamation processes | Cambodia | 1 182 | 1 182 |
| 522 | Gold and silver extraction with mercury amalgamation processes | Philippines | 65 824 | 65 824 |
| 524 | Copper extraction and initial processing | Philippines | 244 | 16 |
| 525 | Lead extraction and initial processing | Philippines | 8 700 | 87 |
| 529 | Primary ferrous metal production | Yemen | 0,03 | 0,03 |
| 529 | Primary ferrous metal production | Madagascar | 40 | 8 |
| 531 | Cement production (a | Madagascar | 40 | 8 |
| 531 | Cement production (a | Yemen | 11 | 52 |
| 531 | Cement production (a | Syria | 521 | 104 |
| 531 | Cement production (a | Philippines | 1 203 | 241 |
| 532 | Pulp and paper production | Philippines | 6 | 6 |
| 533 | Production of lime and light weight aggregates | Cambodia | 0,01 | 0,002 |
| 533 | Production of lime and light weight aggregates | Yemen | 0,01 | 0,01 |
| 533 | Production of lime and light weight aggregates | Madagascar | 0,12 | 0,12 |
| 541 | Chlor-alkali production with mercury-technology | Syria | 1 500 | 100 |
| 551 | Thermometers with mercury | Yemen | 1 | 3 |
| 551 | Thermometers with mercury | Cambodia | 21 | 7 |
| 551 | Thermometers with mercury | Madagascar | 476 | 22 |
| 551 | Thermometers with mercury | Philippines | 198 | 66 |
| 551 | Thermometers with mercury | Burkina Faso | 445 | 148 |
| 551 | Thermometers with mercury | Syria | 750 | 250 |
| 552 | Electrical switches and relays with mercury | Madagascar | 4 486 | 359 |
| 552 | Electrical switches and relays with mercury | Yemen | 460 | 1 610 |
| 552 | Electrical switches and relays with mercury | Philippines | 22 177 | 1 774 |
| 553 | Light sources with mercury | Yemen | 0,01 | 0,03 |
| 553 | Light sources with mercury | Madagascar | 2 | 1 |
| 553 | Light sources with mercury | Burkina Faso | 11 | 6 |
| 553 | Light sources with mercury | Syria | 15 | 15 |
| 553 | Light sources with mercury | Philippines | 343 | 280 |
| 554 | Batteries with mercury | Syria | 18 | 18 |
| 554 | Batteries with mercury | Cambodia | 8 470 | 204 |
| 554 | Batteries with mercury | Burkina Faso | 281 | 281 |
| 554 | Batteries with mercury | Yemen | 483 | 483 |
| 554 | Batteries with mercury | Madagascar | 21 888 | 21 888 |
| 555 | Biocides and pesticides with mercury | Madagascar | 10 100 | 0,00 |
| 556 | Paints with mercury | Yemen | 5 | 25 |
| 556 | Paints with mercury | Madagascar | 1 165 | 70 |
| 557 | Production of pharmaceuticals for human and veterinary uses | Syria | 0,18 | 0,18 |
| 558 | Cosmetics and related products with mercury | Burkina Faso | 100 | 50 |
| 558 | Cosmetics and related products with mercury | Yemen | 50 | 250 |
| 558 | Cosmetics and related products with mercury | Madagascar | 10 450 | 2 090 |
| 561 | Dental mercury-amalgam fillings (b | Madagascar | 3 588 | 0,00 |
| 561 | Dental mercury-amalgam fillings (b | Yemen | 1 | 5 |
| 561 | Dental mercury-amalgam fillings (b | Burkina Faso | 25 | 25 |
| 561 | Dental mercury-amalgam fillings (b | Cambodia | 163 | 163 |
| 561 | Dental mercury-amalgam fillings (b | Syria | 4 200 | 4 200 |
| 561 | Dental mercury-amalgam fillings (b | Philippines | 17 741 | 4 435 |
| 562 | Manometers and gauges with mercury | Yemen | 3 | 3 |
| 562 | Manometers and gauges with mercury | Syria | 1 464 | 345 |
| 563 | Laboratory chemicals | Syria | 326 | 326 |
| 563 | Laboratory chemicals and equipment with mercury | Philippines | 13 105 | 2 184 |
| 565 | Miscellaneous product uses, mercury metal uses, and other sources | Philippines | 22 800 | 22 800 |
| 581 | Incineration of municipal/general waste | Cambodia | 35 | 4 |
| 583 | Incineration of medical waste | Cambodia | 32 | 6 |
| 583 | Incineration of medical waste | Yemen | 3 | 8 |
| 583 | Incineration of medical waste | Syria | 40 | 8 |
| 583 | Incineration of medical waste | Burkina Faso | 41 | 8 |
| 583 | Incineration of medical waste | Madagascar | 507 | 101 |
| 585 | Informal waste incineration | Burkina Faso | 326 | 33 |
| 585 | Informal waste incineration | Syria | 2 422 | 242 |
| 585 | Informal waste incineration | Madagascar | 7 332 | 733 |
| 591 | Controlled landfills/deposits (a | Burkina Faso | 1 985 | 199 |
| 591 | Controlled landfills/deposits (a | Philippines | 4 577 | 458 |
| 591 | Controlled landfills/deposits (a | Cambodia | 4 670 | 467 |
| 591 | Controlled landfills/deposits (a | Syria | 4 678 | 4 678 |
| 593 | Informal local disposal of industrial production waste | Philippines | 4 577 | 458 |
| 593 | Informal local disposal of industrial production waste | Madagascar | 21 573 | 2 164 |
| 594 | Informal dumping of general waste (b | Yemen | 4 745 | 47 |
| 594 | Informal dumping of general waste (b | Burkina Faso | 730 | 73 |
| 595 | Waste water system/treatment | Madagascar | 5 | 0,25 |
| 595 | Waste water system/treatment | Burkina Faso | 25 | 1 |
| 595 | Waste water system/treatment | Philippines | 19 615 | 981 |
| 5101 | Crematoria | Philippines | 153 | 38 |
| 5101 | Crematoria | Cambodia | 162 | 41 |
| 5102 | Cemetaries | Madagascar | 872 | 218 |
| 5102 | Cemetaries | Philippines | 1 377 | 344 |

### 1.5 Output data

1.5.1 General output data (min and max values)

Table 5: General output data (min and max values)

| Max input per country |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Country | Total output | Air | Water | Land | Product | Waste | Disposal |
| Philippines | 190 041 | 96 619 | 34 708 | 31 063 | 4 150 | 20 892 | 2 610 |
| Yemen | 4 774 | 21 | 2 849 | 1 424 | 5 | 476 | 0 |
| Cambodia | 6 200 | 1 052 | 4 437 | 703 | 0 | 8 | 0 |
| Syria | 18 349 | 8 380 | 2 369 | 936 | 2 497 | 4 168 | 0 |
| Burkina Faso | 3 803 | 545 | 1 669 | 1 589 | 0 | 0 | 0 |
| Madagascar | 30 794 | 8 272 | 13 421 | 4 787 | 0 | 4 315 | 0 |
| Total (kg) | 253 962 | 114 889 | 59 452 | 40 501 | 6 652 | 29 857 | 2 610 |
| Min input per country |  |  |  |  |  |  |  |
| Country | Total output | Air | Water | Land | Product | Waste | Disposal |
| Philippines | 125 377 | 78 565 | 14 335 | 15 447 | 289 | 16 715 | 26 |
| Yemen | 1 677 | 1 627 | 30 | 14 | 0 | 6 | 0 |
| Cambodia | 1 702 | 763 | 656 | 283 | 0 | 0 | 0 |
| Syria | 5 974 | 866 | 2 514 | 936 | 429 | 1 230 | 0 |
| Burkina Faso | 728 | 266 | 230 | 232 | 0 | 0 | 0 |
| Madagascar | 3 497 | 1 115 | 1 408 | 542 | 0 | 433 | 0 |
| Total (kg) | 138 955 | 83 201 | 19 172 | 17 454 | 718 | 18 384 | 26 |

1.5.2 Output data per country and  category

Table 6: Output data per country and category - Philippines

|  |  | Min values (kg) |  |  |  |  |  |  |  | Max values (kg) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no | Sub category name | output | Air | Water | Land | Product | Waste | Dispos | Disp | output | Air | Water | Land | Product | Waste | Dispos | Disp |
| 511 | Coal combustion in large power plants | 539 | 485 | 0 | 0 | 0 | 54 | 0 | 0 | 5 386 | 4 847 | 0 | 0 | 0 | 539 | 0 | 0 |
| 513 | Mineral oils - extraction, refining and use | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 37 | 34 | 0 | 0 | 0 | 4 | 0 | 0 |
| 514 | Natural gas - extraction, refining and use | 6 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 577 | 577 | 0 | 0 | 0 | 0 | 0 | 0 |
| 517 | Geothermal power production | 31 395 | 31 395 | 0 | 0 | 0 | 0 | 0 | 0 | 41 860 | 41 860 | 0 | 0 | 0 | 0 | 0 | 0 |
| 522 | Gold and silver extraction with mercury amalgamation processes | 65 824 | 39 494 | 13 165 | 13 165 | 0 | 0 | 0 | 0 | 65 824 | 39 494 | 13 165 | 13 165 | 0 | 0 | 0 | 0 |
| 524 | Copper extraction and initial processing | 16 | 3 | 6 | 6 | 0 | 0 | 0 | 0 | 244 | 49 | 98 | 98 | 0 | 0 | 0 | 0 |
| 525 | Lead extraction and initial processing | 87 | 9 | 0 | 26 | 26 | 0 | 26 | 26 | 8 700 | 870 | 0 | 2 610 | 2 610 | 0 | 2 610 | 2610 |
| 531 | Cement production (a | 241 | 24 | 0 | 0 | 217 | 0 | 0 | 0 | 1 203 | 120 | 0 | 0 | 1 083 | 0 | 0 | 0 |
| 532 | Pulp and paper production | 6 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 6 | 0 | 0 | 0 | 0 | 0 | 0 |
| 551 | Thermometers with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 552 | Electrical switches and relays with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 553 | Light sources with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 561 | Dental mercury-amalgam fillings (b | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 563 | Laboratory chemicals and equipment with mercury | 2 184 | 218 | 0 | 1 310 | 0 | 655 | 0 | 0 | 13 105 | 1 311 | 0 | 7 863 | 0 | 3 932 | 0 | 0 |
| 565 | Miscellaneous product uses, mercury metal uses, and other sources | 22 800 | 6 840 | 0 | 0 | 0 | 15 960 | 0 | 0 | 22 800 | 6 840 | 0 | 0 | 0 | 15 960 | 0 | 0 |
| 591 | Controlled landfills/deposits (a | 458 | 0 | 46 | 412 | 0 | 0 | 0 | 0 | 4 577 | 0 | 458 | 4 119 | 0 | 0 | 0 | 0 |
| 593 | Informal local disposal of industrial production waste | 458 | 46 | 137 | 183 | 46 | 46 | 0 | 0 | 4 577 | 458 | 1 373 | 1 831 | 458 | 458 | 0 | 0 |
| 595 | Waste water system/treatment | 981 | 0 | 981 | 0 | 0 | 0 | 0 | 0 | 19 615 | 0 | 19 615 | 0 | 0 | 0 | 0 | 0 |
| 5101 | Crematoria | 38 | 38 | 0 | 0 | 0 | 0 | 0 | 0 | 153 | 153 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5102 | Cemetaries | 344 | 0 | 0 | 344 | 0 | 0 | 0 | 0 | 1 377 | 0 | 0 | 1 377 | 0 | 0 | 0 | 0 |

Table 7: Output data per country and  category - Yemen

| no | Sub category name | output | Air | Water | Land | Product | Waste | Dispos | Disp | output | Air | Water | Land | Product | Waste | Dispos | Disp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 513 | Mineral oils - extraction, refining and use | 1 564 | 1 564 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 5 | 0 | 0 | 0 | 0 | 0 | 0 |
| 516 | Biomass fired power and heat production | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| 529 | Primary ferrous metal production | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 531 | Cement production (a | 52 | 52 | 0 | 0 | 0 | 0 | 0 | 0 | 11 | 11 | 0 | 0 | 0 | 0 | 0 | 0 |
| 533 | Production of lime and light weight aggregates | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 551 | Thermometers with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 552 | Electrical switches and relays with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |  | 0 | 0 | 0 | 0 |
| 553 | Light sources with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 554 | Batteries with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 556 | Paints with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 0 | 0 | 0 | 5 | 0 | 0 | 0 |
| 558 | Cosmetics and related products with mercury | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 561 | Dental mercury-amalgam fillings (b | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 562 | Manometers and gauges with mercury | 3 | 0 | 2 | 0 | 0 | 2 | 0 | 0 | 3 | 0 | 2 | 0 | 0 | 2 | 0 | 0 |
| 583 | Incineration of medical waste | 8 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| 594 | Informal dumping of general waste (b | 47 | 0 | 28 | 14 | 0 | 5 | 0 | 0 | 4 745 | 0 | 2 847 | 1 424 | 0 | 475 | 0 | 0 |

Table 8: Output data per country and  category - Cambodia

|  |  | Min values (kg) |  |  |  |  |  |  |  |  | Max values (kg) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no | Sub category name |  | output | Air | Water | Land | Product | Waste | Dispos | Disp | output | Air | Water | Land | Product | Waste | Dispos | Disp |
| 513 | Mineral oils - extraction, refining and use |  | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 119 | 112 | 0 | 0 | 0 | 8 | 0 | 0 |
| 514 | Natural gas - extraction, refining and use |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 516 | Biomass fired power and heat production |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 522 | Gold and silver extraction with mercury amalgamation processes |  | 1 182 | 709 | 236 | 236 | 0 | 0 | 0 | 0 | 1 182 | 709 | 236 | 236 | 0 | 0 | 0 | 0 |
| 533 | Production of lime and light weight aggregates |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 551 | Thermometers with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 554 | Batteries with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 561 | Dental mercury-amalgam fillings (b |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 581 | Incineration of municipal/general waste |  | 4 | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 35 | 35 | 0 | 0 | 0 | 0 | 0 | 0 |
| 583 | Incineration of medical waste |  | 6 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 32 | 32 | 0 | 0 | 0 | 0 | 0 | 0 |
| 591 | Controlled landfills/deposits (a |  | 467 | 0 | 420 | 47 | 0 | 0 | 0 | 0 | 4 670 | 2 | 4 201 | 467 | 0 | 0 | 0 | 0 |
| 5101 | Crematoria |  | 41 | 41 | 0 | 0 | 0 | 0 | 0 | 0 | 162 | 162 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 9: Output data per country and  category - Syria

|  |  |  | Min values (kg) |  |  |  |  |  |  |  |  | Max values (kg) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no | Sub category name |  |  | output | Air | Water | Land | Product | Waste | Dispos | Disp |  | output | Air | Water | Land | Product | Waste | Dispos | Disp |
| 513 | Mineral oils - extraction, refining and use |  |  | 138 | 79 | 0 | 0 | 0 | 59 | 0 | 0 |  | 5 542 | 3 780 | 0 | 0 | 0 | 1 762 | 0 | 0 |
| 514 | Natural gas - extraction, refining and use |  |  | 33 | 17 | 0 | 0 | 17 | 0 | 0 | 0 |  | 3 320 | 1 660 | 0 | 0 | 1 660 | 0 | 0 | 0 |
| 516 | Biomass fired power and heat production |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 531 | Cement production (a |  |  | 104 | 18 | 0 | 0 | 87 | 0 | 0 | 0 |  | 521 | 10 | 0 | 0 | 511 | 0 | 0 | 0 |
| 541 | Chlor-alkali production with mercury-technology |  |  | 100 | 0 | 2 | 0 | 0 | 98 | 0 | 0 |  | 1 500 | 0 | 30 | 0 | 0 | 1 470 | 0 | 0 |
| 551 | Thermometers with mercury |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 553 | Light sources with mercury |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 554 | Batteries with mercury |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 557 | Production of pharmaceuticals for human and veterinary uses |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 561 | Dental mercury-amalgam fillings (b |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 562 | Manometers and gauges with mercury |  |  | 345 | 35 | 173 | 0 | 0 | 138 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 563 | Laboratory chemicals |  |  | 326 | 0 | 0 | 0 | 326 | 0 | 0 | 0 |  | 326 | 0 | 0 | 0 | 326 | 0 | 0 | 0 |
| 583 | Incineration of medical waste |  |  | 8 | 8 | 0 | 0 | 0 | 0 | 0 | 0 |  | 40 | 40 | 0 | 0 | 0 | 0 | 0 | 0 |
| 585 | Informal waste incineration |  |  | 242 | 242 | 0 | 0 | 0 | 0 | 0 | 0 |  | 2 422 | 2 422 | 0 | 0 | 0 | 0 | 0 | 0 |
| 591 | Controlled landfills/deposits (a |  |  | 4 678 | 468 | 2 339 | 936 | 0 | 936 | 0 | 0 |  | 4 678 | 468 | 2 339 | 936 | 0 | 936 | 0 | 0 |

Table 10: Output data per country and  category - Burkina Faso

|  |  |  | Min values (kg) |  |  |  |  |  |  |  |  | Max values (kg) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no | Sub category name |  |  | output | Air | Water | Land | Product | Waste | Dispos | Disp | output | Air | Water | Land | Product | Waste | Dispos | Disp |
| 512 | Other coal use |  |  | 13 | 13 | 0 | 0 | 0 | 0 | 0 | 0 | 132 | 132 | 0 | 0 | 0 | 0 | 0 | 0 |
| 513 | Mineral oils - extraction, refining and use |  |  | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 62 | 62 | 0 | 0 | 0 | 0 | 0 | 0 |
| 516 | Biomass fired power and heat production |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 522 | Gold and silver extraction with mercury amalgamation processes |  |  | 400 | 240 | 80 | 80 | 0 | 0 | 0 | 0 | 400 | 240 | 80 | 80 | 0 | 0 | 0 | 0 |
| 551 | Thermometers with mercury |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 553 | Light sources with mercury |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 554 | Batteries with mercury |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 558 | Cosmetics and related products with mercury |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 100 | 2 | 98 | 0 | 0 | 0 | 0 | 0 |
| 561 | Dental mercury-amalgam fillings (b |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 583 | Incineration of medical waste |  |  | 8 | 2 | 3 | 3 | 0 | 0 | 0 | 0 | 41 | 8 | 16 | 16 | 0 | 0 | 0 | 0 |
| 585 | Informal waste incineration |  |  | 33 | 10 | 10 | 13 | 0 | 0 | 0 | 0 | 326 | 98 | 98 | 131 | 0 | 0 | 0 | 0 |
| 591 | Controlled landfills/deposits (a |  |  | 199 | 0 | 99 | 99 | 0 | 0 | 0 | 0 | 1 985 | 1 | 992 | 993 | 0 | 0 | 0 | 0 |
| 594 | Informal dumping of general waste (b |  |  | 73 | 0 | 36 | 37 | 0 | 0 | 0 | 0 | 730 | 0 | 365 | 365 | 0 | 0 | 0 | 0 |
| 595 | Waste water system/treatment |  |  | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 25 | 0 | 20 | 5 | 0 | 0 | 0 | 0 |

Table 11:  Output data per country and  category - Madagascar

|  |  | Min values (kg) |  |  |  |  |  |  |  |  | Max values (kg) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no | Sub category name |  | output | Air | Water | Land | Product | Waste | Dispos | Disp | output | Air | Water | Land | Product | Waste | Dispos | Disp |
| 511 | Coal combustion in large power plants |  | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 17 | 17 | 0 | 0 | 0 | 0 | 0 | 0 |
| 513 | Mineral oils - extraction, refining and use |  | 45 | 45 | 0 | 0 | 0 | 0 | 0 | 0 | 46 | 46 | 0 | 0 | 0 | 0 | 0 | 0 |
| 516 | Biomass fired power and heat production |  | 182 | 182 | 0 | 0 | 0 | 0 | 0 | 0 | 182 | 182 | 0 | 0 | 0 | 0 | 0 | 0 |
| 522 | Gold and silver extraction with mercury amalgamation processes |  | 36 | 36 | 0 | 0 | 0 | 0 | 0 | 0 | 180 | 108 | 36 | 36 | 0 | 0 | 0 | 0 |
| 529 | Primary ferrous metal production |  | 8 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 40 | 40 | 0 | 0 | 0 | 0 | 0 | 0 |
| 531 | Cement production (a |  | 8 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 40 | 40 | 0 | 0 | 0 | 0 | 0 | 0 |
| 533 | Production of lime and light weight aggregates |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 551 | Thermometers with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 552 | Electrical switches and relays with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 553 | Light sources with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 554 | Batteries with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 555 | Biocides and pesticides with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 556 | Paints with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 558 | Cosmetics and related products with mercury |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 561 | Dental mercury-amalgam fillings (b |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 583 | Incineration of medical waste |  | 101 | 101 | 0 | 0 | 0 | 0 | 0 | 0 | 507 | 507 | 0 | 0 | 0 | 0 | 0 | 0 |
| 585 | Informal waste incineration |  | 733 | 733 | 0 | 0 | 0 | 0 | 0 | 0 | 7 332 | 7 332 | 0 | 0 | 0 | 0 | 0 | 0 |
| 593 | Informal local disposal of industrial production waste |  | 2 164 | 0 | 1 298 | 433 | 0 | 433 | 0 | 0 | 21 573 | 0 | 12 944 | 4 315 | 0 | 4 315 | 0 | 0 |
| 595 | Waste water system/treatment |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 0 | 5 | 0 | 0 | 0 | 0 | 0 |
| 5102 | Cemetaries |  | 218 | 0 | 109 | 109 | 0 | 0 | 0 | 0 | 872 | 0 | 436 | 436 | 0 | 0 | 0 | 0 |

### 1.5.3 Mass balance between input and output

The results are based on the mass balance principal.  This is convenient as mercury is an element and can not be changed in form.   Therefore, 100 % of the amount of input should be distributed in the different media.

The following table shows the balance between inputs and outputs.  The highlight fields indicate a positive mass balance corresponding to category 55, illustrating that this aspect has proven challenging in areas with more than one life cycle, such as for mercury in products.

Table 12: Mass balance between input and output

|  |  |  | Min values |  |  | Max values |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Country | no | Sub category name | input | otput | Dif | input | otput | diff |
| Philippines | 511 | Coal combustion in large power plants | 539 | 539 | 0 | 5 386 | 5 386 | 0 |
|  | 513 | Mineral oils - extraction, refining and use | 0 | 0 | 0 | 37 | 37 | 0 |
|  | 514 | Natural gas - extraction, refining and use | 6 | 6 | 0 | 577 | 577 | 0 |
|  | 517 | Geothermal power production | 31 395 | 31 395 | 0 | 41 860 | 41 860 | 0 |
|  | 522 | Gold and silver extraction with mercury amalgamation processes | 65 824 | 65 824 | 0 | 65 824 | 65 824 | 0 |
|  | 524 | Copper extraction and initial processing | 16 | 16 | 0 | 244 | 244 | 0 |
|  | 525 | Lead extraction and initial processing | 87 | 87 | 0 | 8 700 | 8 700 | 0 |
|  | 531 | Cement production (a | 241 | 241 | 0 | 1 203 | 1 203 | 0 |
|  | 532 | Pulp and paper production | 6 | 6 | 0 | 6 | 6 | 0 |
|  | 551 | Thermometers with mercury | 66 | 0 | 66 | 198 | 0 | 198 |
|  | 552 | Electrical switches and relays with mercury | 1 774 | 0 | 1 774 | 22 177 | 0 | 22 177 |
|  | 553 | Light sources with mercury | 280 | 0 | 280 | 343 | 0 | 343 |
|  | 561 | Dental mercury-amalgam fillings (b | 4 435 | 0 | 4 435 | 17 741 | 0 | 17 741 |
|  | 563 | Laboratory chemicals and equipment with mercury | 2 184 | 2 184 | 0 | 13 105 | 13 105 | 0 |
|  | 565 | Miscellaneous product uses, mercury metal uses, and other sources | 22 800 | 22 800 | 0 | 22 800 | 22 800 | 0 |
|  | 591 | Controlled landfills/deposits (a | 458 | 458 | 0 | 4 577 | 4 577 | 0 |
|  | 593 | Informal local disposal of industrial production waste | 458 | 458 | 0 | 4 577 | 4 577 | 0 |
|  | 595 | Waste water system/treatment | 981 | 981 | 0 | 19 615 | 19 615 | 0 |
|  | 5101 | Crematoria | 38 | 38 | 0 | 153 | 153 | 0 |
|  | 5102 | Cemetaries | 344 | 344 | 0 | 1 377 | 1 377 | 0 |
| Yemen | 513 | Mineral oils - extraction, refining and use | 1 564 | 1 564 | 0 | 5 | 5 | 0 |
|  | 516 | Biomass fired power and heat production | 3 | 3 | 0 | 3 | 3 | 0 |
|  | 529 | Primary ferrous metal production | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 531 | Cement production (a | 52 | 52 | 0 | 11 | 11 | 0 |
|  | 533 | Production of lime and light weight aggregates | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 551 | Thermometers with mercury | 3 | 0 | 3 | 1 | 0 | 1 |
|  | 552 | Electrical switches and relays with mercury | 1 610 | 0 | 1 610 | 460 |  | 460 |
|  | 553 | Light sources with mercury | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 554 | Batteries with mercury | 483 | 0 | 483 | 483 | 0 | 483 |
|  | 556 | Paints with mercury | 25 | 0 | 25 | 5 | 5 | 0 |
|  | 558 | Cosmetics and related products with mercury | 250 | 0 | 250 | 50 | 0 | 50 |
|  | 561 | Dental mercury-amalgam fillings (b | 5 | 0 | 5 | 1 | 0 | 1 |
|  | 562 | Manometers and gauges with mercury | 3 | 3 | 0 | 3 | 3 | 0 |
|  | 583 | Incineration of medical waste | 8 | 8 | 0 | 3 | 3 | 0 |
|  | 594 | Informal dumping of general waste (b | 47 | 47 | 0 | 4 745 | 4 745 | 0 |
| Cambodia | 513 | Mineral oils - extraction, refining and use | 3 | 3 | 0 | 119 | 119 | 0 |
|  | 514 | Natural gas - extraction, refining and use | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 516 | Biomass fired power and heat production | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 522 | Gold and silver extraction with mercury amalgamation processes | 1 182 | 1 182 | 0 | 1 182 | 1 182 | 0 |
|  | 533 | Production of lime and light weight aggregates | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 551 | Thermometers with mercury | 7 | 0 | 7 | 21 | 0 | 21 |
|  | 554 | Batteries with mercury | 204 | 0 | 204 | 8 470 | 0 | 8 470 |
|  | 561 | Dental mercury-amalgam fillings (b | 113 | 0 | 113 | 113 | 0 | 113 |
|  | 581 | Incineration of municipal/general waste | 4 | 4 | 0 | 35 | 35 | 0 |
|  | 583 | Incineration of medical waste | 6 | 6 | 0 | 32 | 32 | 0 |
|  | 591 | Controlled landfills/deposits (a | 467 | 467 | 0 | 4 670 | 4 670 | 0 |
|  | 5101 | Crematoria | 41 | 41 | 0 | 162 | 162 | 0 |
| Syria | 513 | Mineral oils - extraction, refining and use | 138 | 138 | 0 | 5 542 | 5 542 | 0 |
|  | 514 | Natural gas - extraction, refining and use | 33 | 33 | 0 | 3 320 | 3 320 | 0 |
|  | 516 | Biomass fired power and heat production | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 531 | Cement production (a | 104 | 104 | 0 | 521 | 521 | 0 |
|  | 541 | Chlor-alkali production with mercury-technology | 100 | 100 | 0 | 1 500 | 1 500 | 0 |
|  | 551 | Thermometers with mercury | 250 | 0 | 250 | 750 | 0 | 750 |
|  | 553 | Light sources with mercury | 15 | 0 | 15 | 15 | 0 | 15 |
|  | 554 | Batteries with mercury | 18 | 0 | 18 | 18 | 0 | 18 |
|  | 557 | Production of pharmaceuticals for human and veterinary uses | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 561 | Dental mercury-amalgam fillings (b | 4 200 | 0 | 4 200 | 4 200 | 0 | 4 200 |
|  | 562 | Manometers and gauges with mercury | 345 | 345 | 0 | 1 464 | 0 | 1 464 |
|  | 563 | Laboratory chemicals | 326 | 326 | 0 | 326 | 326 | 0 |
|  | 583 | Incineration of medical waste | 8 | 8 | 0 | 40 | 40 | 0 |
|  | 585 | Informal waste incineration | 242 | 242 | 0 | 2 422 | 2 422 | 0 |
|  | 591 | Controlled landfills/deposits (a | 4 678 | 4 678 | 0 | 4 678 | 4 678 | 0 |
| Burkina Faso | 512 | Other coal use | 13 | 13 | 0 | 132 | 132 | 0 |
|  | 513 | Mineral oils - extraction, refining and use | 1 | 1 | 0 | 62 | 62 | 0 |
|  | 516 | Biomass fired power and heat production | 0 | 0 | 0 | 1 | 1 | 0 |
|  | 522 | Gold and silver extraction with mercury amalgamation processes | 400 | 400 | 0 | 400 | 400 | 0 |
|  | 551 | Thermometers with mercury | 148 | 0 | 148 | 445 | 0 | 445 |
|  | 553 | Light sources with mercury | 6 | 0 | 6 | 11 | 0 | 11 |
|  | 554 | Batteries with mercury | 281 | 0 | 281 | 281 | 0 | 281 |
|  | 558 | Cosmetics and related products with mercury | 50 | 0 | 50 | 100 | 100 | 0 |
|  | 561 | Dental mercury-amalgam fillings (b | 25 | 0 | 25 | 25 | 0 | 25 |
|  | 583 | Incineration of medical waste | 8 | 8 | 0 | 41 | 41 | 0 |
|  | 585 | Informal waste incineration | 33 | 33 | 0 | 326 | 326 | 0 |
|  | 591 | Controlled landfills/deposits (a | 199 | 199 | 0 | 1 985 | 1 985 | 0 |
|  | 594 | Informal dumping of general waste (b | 73 | 73 | 0 | 730 | 730 | 0 |
|  | 595 | Waste water system/treatment | 1 | 1 | 0 | 25 | 25 | 0 |
| Madagascar | 511 | Coal combustion in large power plants | 2 | 2 | 0 | 17 | 17 | 0 |
|  | 513 | Mineral oils - extraction, refining and use | 45 | 45 | 0 | 46 | 46 | 0 |
|  | 516 | Biomass fired power and heat production | 182 | 182 | 0 | 182 | 182 | 0 |
|  | 522 | Gold and silver extraction with mercury amalgamation processes | 60 | 36 | 24 | 180 | 180 | 0 |
|  | 529 | Primary ferrous metal production | 8 | 8 | 0 | 40 | 40 | 0 |
|  | 531 | Cement production (a | 8 | 8 | 0 | 40 | 40 | 0 |
|  | 533 | Production of lime and light weight aggregates | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 551 | Thermometers with mercury | 22 | 0 | 22 | 476 | 0 | 476 |
|  | 552 | Electrical switches and relays with mercury | 359 | 0 | 359 | 4 486 | 0 | 4 486 |
|  | 553 | Light sources with mercury | 1 | 0 | 1 | 2 | 0 | 2 |
|  | 554 | Batteries with mercury | 21 888 | 0 | 21 888 | 21 888 | 0 | 21 888 |
|  | 555 | Biocides and pesticides with mercury | 0 | 0 | 0 | 10 100 | 0 | 10 100 |
|  | 556 | Paints with mercury | 70 | 0 | 70 | 1 165 | 0 | 1 165 |
|  | 558 | Cosmetics and related products with mercury | 2 090 | 0 | 2 090 | 10 450 | 0 | 10 450 |
|  | 561 | Dental mercury-amalgam fillings (b | 0 | 0 | 0 | 3 588 | 0 | 3 588 |
|  | 583 | Incineration of medical waste | 101 | 101 | 0 | 507 | 507 | 0 |
|  | 585 | Informal waste incineration | 733 | 733 | 0 | 7 332 | 7 332 | 0 |
|  | 593 | Informal local disposal of industrial production waste | 2 164 | 2 164 | 0 | 21 573 | 21 573 | 0 |
|  | 595 | Waste water system/treatment | 0 | 0 | 0 | 5 | 5 | 0 |
|  | 5102 | Cemetaries | 218 | 218 | 0 | 872 | 872 | 0 |

The difference between mi n input and min output is:

| Min Input | Min output | diff |
| --- | --- | --- |
| 177 656 | 138 955 | 38 701 |

## 2.0	Observations and lessons learned in applying the toolkit methodology

### 2.1  Understanding, Structure and Language

Overall feedback on the understanding and structure of the toolkit has been positive.  It was noted that it would be helpful to have brief sector specific information to communicate with sectors in information gathering, including in some cases better definitions (such as for batteries).

Countries generally noted appreciation for the translated versions of the toolkit; however, there is room for improvement in the quality of translations.

### 2.2  Toolkit Categories, Emission factors

It has been noted that the sub-categories sometimes do not relate to national situations and that emission factors should be more relevant for developing countries.  Overall it is felt that reducing the emission factor range would improve usefulness of results.  A need has been identified to improve the emission factors for batteries – currently thought to be too high.

### 2.3 Data gathering

Estimating activity and accessing data is challenging.  It has been noted that the toolkit could benefit from including more strategies to access data, such as simple sample questionnaires.

Generally, the information used by the countries are the following: customs data, inventory of dioxins in the Stockholm Convention, climate change (emission of CO2) and the country profile of SAICM.

For mercury in products, questionnaires can be overwhelming; a market analysis may be a better starting point to explore.

In some cases (eg, fever thermometers) import data is crucial because information on actual

breakage/ disposal of products cannot be found.  Section 5.5 requires a better description and labeling for mercury in products, such as batteries, skin products, lighting, electronic devices in order to assist in data gathering (particularly at the customs level).  The Prior Informed Consent Procedure of the Rotterdam Convention could assist in providing information on mercury and its compounds used as pesticides, but currently, industrial uses of mercury are not covered under the Rotterdam Convention.

### 2.4 –Double Recording

As noted in section 1.5.3, Category 55 (Consumer products containing mercury) has presented some challenges for countries in terms of double counting in the use/disposal phase.

The tables below gives the categories for the 2 groups of categories which are concerned by the double recording.  Consideration to this area will be given in the toolkit review process.

Category 55 : Consumer products containing mercury

Table 13: Category 55 : Consumer products containing mercury

| no | Sub category name | Phase 1 | Phase 2 |
| --- | --- | --- | --- |
| 551 | Thermometers with mercury | /Use+disposal : | Medical thermometers |
| 551 | Thermometers with mercury | /Use+disposal : | Ambient air thermom. |
| 551 | Thermometers with mercury | /Use+disposal : | Industrial and special th. |
| 551 | Thermometers with mercury | /Use+disposal : | Other glass Hg thermometers |
| 552 | Electrical switches and relays with mercury | /Use+disposal : |  |
| 553 | Light sources with mercury | /Use+disposal : | Fluorescent tubes (double end) |
| 553 | Light sources with mercury | /Use+disposal : | Compact fluorescent lamp (CFL single end) |
| 553 | Light sources with mercury | /Use+disposal : | High pressure mercury vapour |
| 553 | Light sources with mercury | /Use+disposal : | High-pressure sodium lamps |
| 553 | Light sources with mercury | /Use+disposal : | UV light for tanning |
| 553 | Light sources with mercury | /Use+disposal : | Metal halide lamps |
| 554 | Batteries with mercury | /Use+disposal : | Mercury oxide (all sizes); also called mercury-zinc cells |
| 554 | Batteries with mercury | /Use+disposal : | Zinc-air button cells |
| 554 | Batteries with mercury | /Use+disposal : | Alkaline button cells |
| 554 | Batteries with mercury | /Use+disposal : | Silver oxide button cells |
| 554 | Batteries with mercury | /Use+disposal : | Alkaline, other than button cell shapes |
| 555 | Biocides and pesticides with mercury | /Use+disposal |  |
| 556 | Paints with mercury | /Use+disposal |  |
| 558 | Cosmetics and related products with mercury | /Use+disposal |  |
| 557 | Production of pharmaceuticals for human and veterinary uses |  |  |
| 554 | Batteries with mercury | /Use+disposal : |  |

Table 14: Areas where double counting can come up in categories 58 & 59

| No Cat | Category name | Sub cat no | Sub category name | Output scenario |
| --- | --- | --- | --- | --- |
| 58 | Waste incineration | 581 | Incineration of municipal/general waste | No emission reduction devices |
| 58 |  | 581 |  | PM reduc, simple ESP, or similar |
| 58 |  | 581 |  | Acid gas control with limestone (or similar acid gas absorbent) and downstream high efficiency FF or ESP PM retention |
| 58 |  | 581 |  | Mercury specific absorbents and downstream FF |
| 58 |  | 582 | Incineration of hazardous waste | No emission reduction devices |
| 58 |  | 582 |  | PM reduc, simple ESP, or similar |
| 58 |  | 582 |  | Acid gas control with limestone (or similar acid gas absorbent) and downstream high efficiency FF or ESP PM retention |
| 58 |  | 582 |  | Mercury specific absorbents and downstream FF |
| 58 |  | 583 | Incineration of medical waste | No emission reduction devices |
| 58 |  | 583 |  | PM reduc, simple ESP, or similar |
| 58 |  | 583 |  | Acid gas control with limestone (or similar acid gas absorbent) and downstream high efficiency FF or ESP PM retention |
| 58 |  | 583 |  | Mercury specific absorbents and downstream FF |
| 58 |  | 584 | Sewage sludge incineration |  |
| 58 |  | 585 | Informal waste incineration |  |
| 59 | Waste deposition landfilling and waste water treatment | 591 | Controlled landfills/deposits (a | (a |
| 59 |  | 592 | Diffuse disposal under some control |  |
| 59 |  | 593 | Informal local disposal of industrial production waste |  |
| 59 |  | 594 | Informal dumping of general waste (b |  |
| 59 |  | 595 | Waste water system/treatment | No treatment; direct release from sewage pipe |
| 59 |  | 595 |  | Mechanical treatment only |
| 59 |  | 595 |  | Mechanical and biological (activated sludge) treatment; no land application of sludge |
| 59 |  | 595 |  | Mechanical and biological (activated sludge) treatment; 40% of sludge used for land application |

__________________

* 	UNEP(DTIE)/Hg/OEWG.2/1.

Results are not yet available for Pakistan.